# Master Assembly & Wiring Guide

# Wireless Weightlifting Trolley Prototype

IMPORTANT SAFETY WARNING:

Before connecting the ESP32 or Relay Modules, you MUST calibrate the Buck Converter to 5.0V using a multimeter. Connecting 24V directly to the logic pins will destroy your electronics instantly.

## Step 1: Power Setup & Calibration

1. Connect the AC Power Cord to the SMPS (L, N, G terminals).

2. Connect SMPS V+ to LM2596 IN+ and SMPS V- to LM2596 IN-.

3. Turn on power and use a Multimeter to tune the LM2596 output to EXACTLY 5.0V.

## Step 2: Logic Power Loop (5V)

1. Connect all Grounds (ESP32 GND, Both Relay GNDs, Buck OUT-) to a common rail.

2. Connect Buck OUT+ (5V) to ESP32 VIN, 4-Ch Relay VCC, and 2-Ch Relay VCC.

## Step 3: Control Signals (ESP32 to Relays)

Connect the following GPIO pins to the Relay Module IN pins:

- GPIO 12 -> 4-Ch Relay IN1 (Lift Up)

- GPIO 13 -> 4-Ch Relay IN2 (Lift Down)

- GPIO 14 -> 4-Ch Relay IN3 (Left Forward)

- GPIO 27 -> 4-Ch Relay IN4 (Left Backward)

- GPIO 26 -> 2-Ch Relay IN1 (Right Forward)

- GPIO 25 -> 2-Ch Relay IN2 (Right Backward)

## Step 4: High-Power Loop (24V)

For all 6 Relays:

1. Connect SMPS 24V+ to the Normally Open (NO) terminal.

2. Connect SMPS GND to the Normally Closed (NC) terminal.

3. Connect Motor wires to the Center (COM) terminals of the respective relay pairs.

## Step 5: Software & Testing

1. Connect to WiFi: Trolley_Pro (Pass: 12345678).

2. Navigate to 192.168.4.1 in your mobile browser.

3. If a motor spins backward, swap its two wires at the relay COM terminals.

## Step 6: Programming the ESP32

1. Open Arduino IDE and select your ESP32 Board.

2. Copy and paste the following code into a new sketch:

#include <WiFi.h>
#include <WebServer.h>

const int liftUpPin = 12; const int liftDownPin = 13;
const int leftFwdPin = 14; const int leftBwdPin = 27;
const int rightFwdPin = 26; const int rightBwdPin = 25;

WebServer server(80);

const char* htmlPage = R"rawliteral(
<!DOCTYPE html><html><head><title>Trolley Command Center</title>
<meta name="viewport" content="width=device-width, initial-scale=1, maximum-scale=1, user-scalable=no">
<style>
  body { text-align:center; font-family:sans-serif; background-color:#121212; color:white; margin:0; }
  .box { padding: 15px; border: 1px solid #333; margin: 10px; border-radius: 12px; }
  button { width: 42%; height: 75px; font-size: 20px; margin: 6px; border-radius: 12px;
           background-color: #007bff; color: white; border: none; font-weight: bold; }
  .nav { background-color: #28a745; }
  .stop { background-color: #dc3545; width: 88%; height: 60px; }
  button:active { background-color: #0056b3; transform: scale(0.96); }
</style></head>
<body>
  <h1>TROLLEY CONTROL</h1>
  <div class="box"><h3>LIFT</h3>
    <button onmousedown="fetch('/liftup')" onmouseup="fetch('/stoplift')">LIFT UP</button>
    <button onmousedown="fetch('/liftdown')" onmouseup="fetch('/stoplift')">LOWER DOWN</button>
  </div>
  <div class="box"><h3>NAVIGATION</h3>
    <button class="nav" onmousedown="fetch('/fwd')" onmouseup="fetch('/stopnav')">FWD</button><br>
    <button class="nav" onmousedown="fetch('/left')" onmouseup="fetch('/stopnav')">LEFT</button>
    <button class="nav" onmousedown="fetch('/right')" onmouseup="fetch('/stopnav')">RIGHT</button><br>
    <button class="nav" onmousedown="fetch('/bwd')" onmouseup="fetch('/stopnav')">BWD</button>
  </div>
  <button class="stop" onclick="fetch('/stopall')">EMERGENCY ALL STOP</button>
</body></html>
)rawliteral";

void handleRoot() { server.send(200, "text/html", htmlPage); }

// Relay Active Low Logic
void stopLift() { digitalWrite(liftUpPin, HIGH); digitalWrite(liftDownPin, HIGH); server.send(200); }
void stopNav() {
  digitalWrite(leftFwdPin, HIGH); digitalWrite(leftBwdPin, HIGH);
  digitalWrite(rightFwdPin, HIGH); digitalWrite(rightBwdPin, HIGH);
  server.send(200);
}

void liftUp() { digitalWrite(liftUpPin, LOW); digitalWrite(liftDownPin, HIGH); server.send(200); }
void liftDown() { digitalWrite(liftUpPin, HIGH); digitalWrite(liftDownPin, LOW); server.send(200); }
void forward() {
  digitalWrite(leftFwdPin, LOW); digitalWrite(leftBwdPin, HIGH);
  digitalWrite(rightFwdPin, LOW); digitalWrite(rightBwdPin, HIGH);
  server.send(200);
}
void backward() {
  digitalWrite(leftFwdPin, HIGH); digitalWrite(leftBwdPin, LOW);
  digitalWrite(rightFwdPin, HIGH); digitalWrite(rightBwdPin, LOW);
  server.send(200);
}
void turnLeft() {
  digitalWrite(leftFwdPin, HIGH); digitalWrite(leftBwdPin, LOW);
  digitalWrite(rightFwdPin, LOW); digitalWrite(rightBwdPin, HIGH);
  server.send(200);
}
void turnRight() {
  digitalWrite(leftFwdPin, LOW); digitalWrite(leftBwdPin, HIGH);
  digitalWrite(rightFwdPin, HIGH); digitalWrite(rightBwdPin, LOW);
  server.send(200);
}

void setup() {
  int pins[] = {12, 13, 14, 27, 26, 25};
  for(int p : pins) { pinMode(p, OUTPUT); digitalWrite(p, HIGH); }
  WiFi.softAP("Trolley_Pro", "12345678");
  server.on("/", handleRoot);
  server.on("/liftup", liftUp); server.on("/liftdown", liftDown); server.on("/stoplift", stopLift);
  server.on("/fwd", forward); server.on("/bwd", backward);
  server.on("/left", turnLeft); server.on("/right", turnRight);
  server.on("/stopnav", stopNav); server.on("/stopall", [](){ stopLift(); stopNav(); });
  server.begin();
}

void loop() { server.handleClient(); }