# Wireless Weightlifting Machine (Trolley) - Project Report

# 1. Project Overview

The project aims to develop a wireless weightlifting trolley remotely controlled by a mobile application. The machine is designed to lift and transport heavy weights, providing a versatile and efficient solution for mechanical material handling.

# 2. Existing Project Assets

Based on the technical assets provided:

Mechanical Design: CAD model showing a 4-wheel chassis with an integrated lifting mechanism (pulley and belt driven).

Motors: 24V 27RPM High Torque DC Worm Gear Motors (Model 5840-31ZY). Self-locking feature ensures safety during lifting.

Power Source: 24V 15A 360W SMPS Regulated Switching Power Supply.

Current Prototype State: Hardware assembly and motor setup are completed.

# 3. Required Components for Integration

The following components are essential to complete the wireless control and power management system:

4x Relays: For high-power switching or as safety interlocks.

Buck Converter: To step down 24V from SMPS to 5V/3.3V for the control logic.

ESP32 Microcontroller: For WiFi/Bluetooth connectivity and mobile app interface.

24V Motor Drivers (e.g., BTS7960 43A): Capable of handling the high stall current (3.1A) of the motors.

Wiring: 18 Gauge Wires (15m) for power and Jumper wires for logic signals.

15A SMPS: (Identified, confirmed for procurement).

# 4. Strategic Suggestions for Success

Motor Selection & Control: Use High-Current H-Bridges (BTS7960) to avoid overheating during heavy lifts. Ensure the motor driver can handle the stall torque requirements.

Safety Interlocks: Install Limit Switches at the top and bottom of the lifting mechanism to prevent mechanical failure from over-travel.

Power Isolation: Use the Buck converter with proper filtering to prevent electrical noise from the motors from resetting the ESP32.

Remote Interface: Develop the mobile application using Blynk IoT or a local Web Server hosted on the ESP32 for low-latency control.

Feedback Systems: Consider adding a Load Cell (HX711) to display the weight being lifted on the mobile app—this will be highly impressive for faculty presentations.

Emergency Stop: Implement both a physical E-Stop and a software 'failsafe' that stops all motors if the WiFi connection is lost.

# 5. Presentation Tips for Faculty

Focus on the 'Autonomous/Remote' aspect. Highlight the self-locking nature of worm gear motors as a key safety feature. Demonstrate the power efficiency of using a regulated 24V system over standard 12V.