# Wireless Weightlifting Trolley - Final Project Documentation

# 1. Project Overview

This project involves the design and development of a wireless, remotely controlled weightlifting machine (Trolley). The system uses high-torque 24V DC worm gear motors for lifting and a 24V 15A SMPS for stable power delivery. The core control is handled by an ESP32 microcontroller, allowing for mobile application integration.

# 2. System Components

Primary Power: 24V 15A 360W SMPS (Regulated Switching Power Supply).

Control Brain: ESP32 Development Board (WiFi + Bluetooth).

Lifting Motors: 24V 27RPM High Torque DC Worm Gear Motors (Self-locking).

Motor Drivers: 24V High-Current H-Bridges (Recommended: BTS7960 43A).

Power Management: DC-DC Buck Converter (24V to 5V for ESP32/Relays).

Safety/Logic: 4-Channel Relay Module (High-power safety interlocks).

Wiring: 18 Gauge Power Wires (15m) and Logic Jumper Wires.

# 3. Hardware Wiring & Connections

Power Input: SMPS 24V (+) and (-) to Motor Driver Power Inputs.

Logic Power: SMPS 24V (+) to Buck Converter Input (+). Buck Converter Output set to 5V.

ESP32 Power: Buck Converter 5V Output to ESP32 'VIN' pin and 'GND'.

Control Signals: ESP32 GPIO 12 to Motor Driver R_PWM (Forward/Up).

Control Signals: ESP32 GPIO 13 to Motor Driver L_PWM (Backward/Down).

Common Ground: Ensure all GND pins (ESP32, Buck Converter, Motor Driver) are connected together.

# 4. Software Setup Instructions

Install Arduino IDE from arduino.cc.

Go to File > Preferences. Add ESP32 URL: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

Go to Tools > Board > Boards Manager. Search for 'ESP32' and Install.

Select Board: 'DOIT ESP32 DEVKIT V1'.

Ensure the Micro-USB cable supports data transfer (not just charging).

# 5. Prototype Testing Code (Web Control)

The following code creates a WiFi Hotspot named 'Trolley_Control'. Access the control panel via 192.168.4.1 in any mobile browser.

#include <WiFi.h>
#include <WebServer.h>

const int motorForward = 12; // UP
const int motorBackward = 13; // DOWN
WebServer server(80);

void setup() {
  pinMode(motorForward, OUTPUT);
  pinMode(motorBackward, OUTPUT);
  WiFi.softAP("Trolley_Control", "12345678");
  server.on("/", [](){ server.send(200, "text/html", "<h1>Trolley Active</h1><a href='/up'>UP</a><br><a href='/down'>DOWN</a><br><a href='/stop'>STOP</a>"); });
  server.on("/up", [](){ digitalWrite(motorForward, HIGH); digitalWrite(motorBackward, LOW); server.send(200); });
  server.on("/down", [](){ digitalWrite(motorForward, LOW); digitalWrite(motorBackward, HIGH); server.send(200); });
  server.on("/stop", [](){ digitalWrite(motorForward, LOW); digitalWrite(motorBackward, LOW); server.send(200); });
  server.begin();
}

void loop() { server.handleClient(); }

# 6. Safety & Future-Proofing Recommendations

Limit Switches: To stop the lift automatically at the top and bottom.

Ultrasonic Sensors: For autonomous obstacle avoidance.

Load Cell (HX711): To display the weight being lifted on the mobile app.

Current Sensing: To detect motor overload and shut down automatically.